GROUP PROJECTS

TCN2141 – COMPUTER NETWORKS

TRIMESTER 1, 2023/2024 (TERM 2310)

Lecture Section: CN2

Submission Date: 1 FEB 2024

| No | Student ID | Student Name | MAJOR |
| --- | --- | --- | --- |
|  | 1221302787 | MOHAMMAD AMIN BIN AYOB | ST |
|  | 1211100319 | MUHAMMAD NURHARIZFAKHRI BIN SHAMSUDDIN | ST |
|  | 1221302732 | HANI HAMIZAH BINTI HAMZAN | ST |

Table of Contents

PART A: ENTREPRENEURIAL SECTION	2

1.	PROJECT IMPLEMENTATION	2

1.1.	GANTT CHART	2

1.2.	PROJECT BUDGET	3

PART B: DESIGN, SETUP AND CONFIGURATION	5

2.	NETWORK TOPOLOGY	5

2.1.	LOGICAL TOPOLOGY	5

2.2.	PHYSICAL TOPOLOGY	6

3.	LOGICAL DESIGN	7

3.1.	ADDRESSING TABLE	7

4.	ROUTER CONFIGURATION	15

5.	IPV6 ROUTER CONFIGURATION	23

6.	DHCP SERVER CONFIGURATION	27

7.	OSPF CONFIGURATION	31

8.	CLOUD AND ISP ROUTER CONFIGURATION	33

9.	ACCESS CONTROL LIST	35

9.1.	DENY INTERNET ACCESS TO ADMIN AND TUTOR ROOM	35

9.2.	DENY INTERNET ACCESS TO WIRELESS NETWORK	36

10.	WEB SERVER CONFIGURATION	37

11.	FTP SERVER CONFIGURATION	39

12.	DNS SERVER CONFIGURATION	43

13.	TM SERVER CONFIGURATION	45

14.	CONNECTIVITY STATUS	48

# PART A: ENTREPRENEURIAL SECTION

## PROJECT IMPLEMENTATION

### GANTT CHART

| Task | October |  |  |  | November |  |  |  | December |  |  |  | January |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | W1 | W2 | W3 | W4 | W1 | W2 | W3 | W4 | W1 | W2 | W3 | W4 | W1 | W2 | W3 | W4 |
| 1.0 Preparation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Task distribution |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.0 Planning |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identifying Users |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Possible Hardwares |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of Networks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Topology Arrangement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.0 Design |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network Analysis |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Logical Design |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Physical Design |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Topology Mockup |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.0 Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Configuration Planning |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network Setup |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Testing and Troubleshooting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.0 Report |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Configuration Documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Report Finalization |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Video demo |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Submission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### PROJECT BUDGET

| COST ANALYSIS |  |  |  |
| --- | --- | --- | --- |
| A. NETWROK HARDWARE AND COMPONENT |  |  |  |
| Product | Price (RM) | QTY | Total (RM) |
| S3910-24TF, 24-Port Gigabit Ethernet L2+ Switch | RM1,709.00 | 7 | RM11,963.00 |
| Cisco 2901 Router ISR G2 CISCO2901/K9 | RM2,620.00 | 1 | RM2,620.00 |
| Cisco 2911 Integrated Services Router (2911 Router) | RM1,000.00 | 4 | RM4,000.00 |
| Student Workstation | RM3,500.00 | 48 | RM168,000.00 |
| Administrator/ Tutor/Instructor Workstation | RM5,000.00 | 24 | RM120,000.00 |
| PowerEdge R7515 Rack Server | RM18,910.14 | 1 | RM18,910.14 |
| Refurbished Cisco Access Point AIR-CAP3702i-C-K9 | RM1,900.00 | 1 | RM1,900.00 |
| GrowV 19' Floor Stand Rack Server Rack 24U (Perforated) (P/G2480FS) | RM1,250.00 | 3 | RM3,750.00 |
| FortiGate-80E Hardware plus 1 Year 24x7 FortiCare and FortiGuard Unified Threat Protection (UTP) | RM7,000.00 | 1 | RM7,000.00 |
| HP LaserJet P3015 (CE528A) | RM1,097.00 | 3 | RM3,291.00 |
| Seagate Skyhawk AI 16TB 3.5" HDD Surveillance NVR Hard Drive SATA 7200RPM Internal Hard Disk | RM1,165.00 | 7 | RM8,155.00 |
| Subtotal |  |  | RM349,589.14 |
| B. Network software and Configuration |  |  |  |
| Product | Price (RM) | Qty | Total (RM) |
| Microsoft Windows Server 2019 Standard OEM PACK Standard 64- Bit License (16 Core, OEM) (2 CPUs / 2VM) (P73- 07788) | RM3,500.00 | 1 | RM 3,500.00 |
| Microsoft Windows 11 License | RM 344.00 | 72 | RM24,768.00 |
| PRTG Network Monitor | RM - | 1 | RM - |
| Ubuntu 18.04 LTS | RM - | 1 | RM - |
| Vmware Workstation 17.5 | RM 42.77 | 1 | RM 42.77 |
| Subtotal |  |  | RM28,310.77 |
| C. Cabling and Sockets components |  |  |  |
| Product | Price (RM) | Qty | Total (RM) |
| CAT-6 Ethernet Cable (30M) | RM 45.00 | 7 | RM 315.00 |
| RJ45 Cat6 Network Plugs/ Connector | RM 50.00 | 70 | RM 3,500.00 |
| RS232 Serial DB9 9Pin Male to Male Extension Cable (5M) | RM 30.00 | 6 | RM 180.00 |
| Subtotal |  |  | RM 3,995.00 |
| D. Man-hour requirement |  |  |  |
| Task | Rate per Hour (RM) | Qty | Total (RM) |
| Project Manager | RM 30.00 | 320 | RM 9,600.00 |
| Network Engineer | RM 40.00 | 320 | RM12,800.00 |
| Technician | RM 15.00 | 250 | RM 3,750.00 |
| Subtotal |  |  | RM26,150.00 |

Table 1 Cost Analysis table

# PART B: DESIGN, SETUP AND CONFIGURATION

## NETWORK TOPOLOGY

### LOGICAL TOPOLOGY

Figure 1- Logical Topology

### PHYSICAL TOPOLOGY

Figure 2 Physical Topology

## LOGICAL DESIGN

### ADDRESSING TABLE

| LAB 1 IPV6 ADDRESS |  |  |  |  |
| --- | --- | --- | --- | --- |
| PC1_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/1 | 2023:2130:2787:3:BD15:91E1:835F:835F | FE80::3:1 |
| PC2_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/2 | 2023:2130:2787:3:9166:7632:68A0:5A1E | FE80::3:1 |
| PC3_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/3 | 2023:2130:2787:3:878B:79E8:6B56:5DB4 | FE80::3:1 |
| PC4_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/4 | 2023:2130:2787:3:E03F:D29D:C40B:B669 | FE80::3:1 |
| PC5_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/5 | 2023:2130:2787:3:998C:8BFA:7D58:6FC5 | FE80::3:1 |
| PC6_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/6 | 2023:2130:2787:3:F729:A23E:94AC:860A | FE80::3:1 |
| PC7_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/7 | 2023:2130:2787:3:91FD:6736:5994:4B02 | FE80::3:1 |
| PC8_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/8 | 2023:2130:2787:3:51CF:35AB:2709:2709 | FE80::3:1 |
| PC9_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/9 | 2023:2130:2787:3:A87C:9AEA:7FB6:7FB6 | FE80::3:1 |
| PC10_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/10 | 2023:2130:2787:3:E71D:D97B:BD57:BD57 | FE80::3:1 |
| PC11_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/11 | 2023:2130:2787:3:5411:38ED:2A5B:2A5B | FE80::3:1 |
| PC12_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/12 | 2023:2130:2787:3:A699:9807:7DD3:7DD3 | FE80::3:1 |
| PC13_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/13 | 2023:2130:2787:3:373B:29A9:1B07:E75 | FE80::3:1 |
| PC14_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/14 | 2023:2130:2787:3:6EFA:42D6:3433:3433 | FE80::3:1 |
| PC15_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/15 | 2023:2130:2787:3:E442:ACEA:9E58:80B6 | FE80::3:1 |
| PC16_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/16 | 2023:2130:2787:3:A23D:949B:8609:7966 | FE80::3:1 |
| PC17_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/17 | 2023:2130:2787:3:68E:EB6A:DDC8:CF36 | FE80::3:1 |
| PC18_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/18 | 2023:2130:2787:3:EFE9:C3B4:B512:B512 | FE80::3:1 |
| PC19_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/19 | 2023:2130:2787:3:386D:1C38:EA6:F104 | FE80::3:1 |
| PC20_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/20 | 2023:2130:2787:3:5B2D:3FF9:2157:13C5 | FE80::3:1 |
| PC21_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/21 | 2023:2130:2787:3:686:EA52:DCC0:CE2E | FE80::3:1 |
| PC22_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/22 | 2023:2130:2787:3:E14D:D3AB:C519:B787 | FE80::3:1 |
| PC23_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/23 | 2023:2130:2787:3:B8A:C332:B590:B590 | FE80::3:1 |
| PC24_LAB1 | FastEthernet 0 | SWITCH_LAB1 FastEthernet 0/24 | 2023:2130:2787:3:89D3:7B31:6DAF:5F0C | FE80::3:1 |
| PC25_LAB1_INSTRUCTOR | FastEthernet 0 | SWITCH_LAB1 GigabitEthernet 0/1 | 2023:2130:2787:3:D6F1:AC3B:8107:8107 | FE80::3:1 |

| LAB 2 IPV6 ADDRESS |  |  |  |  |
| --- | --- | --- | --- | --- |
| PC1_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/1 | 2023:2130:2787:5:87A3:7901:5DDC:5DDC | FE80::4:1 |
| PC2_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/2 | 2023:2130:2787:5:F203:D6DF:D6DF:C83D | FE80::4:1 |
| PC3_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/3 | 2023:2130:2787:5:58CE:1076:2D3:F441 | FE80::4:1 |
| PC4_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/4 | 2023:2130:2787:5:5548:1DEF:9FDD:813A | FE80::4:1 |
| PC5_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/5 | 2023:2130:2787:5:65A6:5814:4A71:3CEF | FE80::4:1 |
| PC6_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/6 | 2023:2130:2787:5:91AF:757B:67E9:5956 | FE80::4:1 |
| PC7_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/7 | 2023:2130:2787:5:9907:8B65:7ED3:6031 | FE80::4:1 |
| PC8_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/8 | 2023:2130:2787:5:68A:F8E8:EA55:DCB3 | FE80::4:1 |
| PC9_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/9 | 2023:2130:2787:5:B530:9A0B:8C69:7ED7 | FE80::4:1 |
| PC10_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/10 | 2023:2130:2787:5:595C:4BBA:3D27:3D27 | FE80::4:1 |
| PC11_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/11 | 2023:2130:2787:5:8CA9:52E3:4441:37BF | FE80::4:1 |
| PC12_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/12 | 2023:2130:2787:5:A603:8AEE:7C4C:7C4C | FE80::4:1 |
| PC13_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/13 | 2023:2130:2787:5:5050:42CE:342C:2699 | FE80::4:1 |
| PC14_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/14 | 2023:2130:2787:5:CF76:A342:95B0:95B0 | FE80::4:1 |
| PC15_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/15 | 2023:2130:2787:5:9341:5BF8:4D56:3FC4 | FE80::4:1 |
| PC16_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/16 | 2023:2130:2787:5:F610:DAEC:DAEC:CC5A | FE80::4:1 |
| PC17_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/17 | 2023:2130:2787:5:EE61:D0DE:C33C:C33C | FE80::4:1 |
| PC18_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/18 | 2023:2130:2787:5:77E3:6951:5BBF:4D2C | FE80::4:1 |
| PC19_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/19 | 2023:2130:2787:5:A616:9874:8AE2:7C50 | FE80::4:1 |
| PC20_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/20 | 2023:2130:2787:5:6F8:EAC4:CEAF:CEAF | FE80::4:1 |
| PC21_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/21 | 2023:2130:2787:5:C61D:AAE8:9C56:8EC4 | FE80::4:1 |
| PC22_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/22 | 2023:2130:2787:5:624B:4617:3885:3885 | FE80::4:1 |
| PC23_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/23 | 2023:2130:2787:5:9901:8B7F:7DDD:7DDD | FE80::4:1 |
| PC24_LAB2 | FastEthernet 0 | SWITCH_LAB2 FastEthernet 0/24 | 2023:2130:2787:5:E0F7:D265:C4D3:B631 | FE80::4:1 |
| PC25_LAB2_INSTRUCTOR | FastEthernet 0 | SWITCH_LAB2 GigabitEthernet 0/1 | 2023:2130:2787:5:128F:DA27:CC95:BEF2 | FE80::4:1 |

| TUTOR ROOM IPV6 ADDRESS |  |  |  |  |
| --- | --- | --- | --- | --- |
| PC_TUTOR1 | FastEthernet 0 | SWITCH_TUTOR_ADMIN FastEthernet 0/1 | 2023:2130:2787:8:3FC9:6F2:EADE:EADE | FE80::9:1 |
| PC_TUTOR2 | FastEthernet 0 | SWITCH_TUTOR_ADMIN FastEthernet 0/2 | 2023:2130:2787:8:188D:AEB:FC59:EEC7 | FE80::9:1 |
| PC_TUTOR3 | FastEthernet 0 | SWITCH_TUTOR_ADMIN FastEthernet 0/3 | 2023:2130:2787:8:2F2D:E8C4:DA32:CC90 | FE80::9:1 |
| PC_TUTOR4 | FastEthernet 0 | SWITCH_TUTOR_ADMIN FastEthernet 0/4 | 2023:2130:2787:8:D60C:BAE8:AC46:9EB4 | FE80::9:1 |
| PC_TUTOR5 | FastEthernet 0 | SWITCH_TUTOR_ADMIN FastEthernet 0/5 | 2023:2130:2787:8:329F:166B:8D9:FA37 | FE80::9:1 |
| PC_TUTOR6 | FastEthernet 0 | SWITCH_TUTOR_ADMIN FastEthernet 0/6 | 2023:2130:2787:8:A78:FCD6:EE44:D0B2 | FE80::9:1 |
| PC_TUTOR7 | FastEthernet 0 | SWITCH_TUTOR_ADMIN FastEthernet 0/7 | 2023:2130:2787:8:470:F6DE:E84C:DAA9 | FE80::9:1 |
| ADMIN OFFICE IPV6 ADDRESS |  |  |  |  |
| ADMIN_1 | FastEthernet 0 | SWITCH_TUTOR_ADMIN FastEthernet 0/8 | 2023:2130:2787:5:68A:F8E8:EA55:DCB3 | FE80::4:1 |
| ADMIN_2 | FastEthernet 0 | SWITCH_TUTOR_ADMIN FastEthernet 0/9 | 2023:2130:2787:5:B530:9A0B:8C69:7ED7 | FE80::4:1 |
| ADMIN_3 | FastEthernet 0 | SWITCH_TUTOR_ADMIN FastEthernet 0/10 | 2023:2130:2787:5:595C:4BBA:3D27:3D27 | FE80::4:1 |
| ADMIN_4 | FastEthernet 0 | SWITCH_TUTOR_ADMIN FastEthernet 0/11 | 2023:2130:2787:5:8CA9:52E3:4441:37BF | FE80::4:1 |
| ADMIN_5 | FastEthernet 0 | SWITCH_TUTOR_ADMIN FastEthernet 0/12 | 2023:2130:2787:5:A603:8AEE:7C4C:7C4C | FE80::4:1 |
| ADMIN_6 | FastEthernet 0 | SWITCH_TUTOR_ADMIN FastEthernet 0/13 | 2023:2130:2787:5:5050:42CE:342C:2699 | FE80::4:1 |
| ADMIN_7 | FastEthernet 0 | SWITCH_TUTOR_ADMIN FastEthernet 0/14 | 2023:2130:2787:5:CF76:A342:95B0:95B0 | FE80::4:1 |

| SYSTEM ADMIN ROOM IPV6 ADDRESS |  |  |  |  |
| --- | --- | --- | --- | --- |
| SYSADMIN1 | FastEthernet 0 | SWITCH_CLASS_SYSADMIN FastEthernet 0/1 | 2023:2130:2787:7:FA60:DE3C:C0AA:C0AA | FE80::8:1 |
| SYSADMIN2 | FastEthernet 0 | SWITCH_CLASS_SYSADMIN FastEthernet 0/2 | 2023:2130:2787:7:BF38:A1A6:9314:8572 | FE80::8:1 |
| CLASS IPV6 ADDRESS |  |  |  |  |
| CLASS_1 | FastEthernet 0 | SWITCH_CLASS_SYSADMIN FastEthernet 0/9 | 2023:2130:2787:7:1F5B:F327:E594:E594 | FE80::8:1 |
| CLASS_2 | FastEthernet 0 | SWITCH_CLASS_SYSADMIN FastEthernet 0/8 | 2023:2130:2787:7:3588:1954:BC2:FD20 | FE80::8:1 |
| CLASS_3 | FastEthernet 0 | SWITCH_CLASS_SYSADMIN FastEthernet 0/7 | 2023:2130:2787:7:6077:53D5:4542:4542 | FE80::8:1 |
| CLASS_4 | FastEthernet 0 | SWITCH_CLASS_SYSADMIN FastEthernet 0/6 | 2023:2130:2787:7:5101:437E:35EC:274A | FE80::8:1 |
| CLASS_5 | FastEthernet 0 | SWITCH_CLASS_SYSADMIN FastEthernet 0/5 | 2023:2130:2787:7:472D:2BF9:C86C:BADA | FE80::8:1 |
| CLASS_6 | FastEthernet 0 | SWITCH_CLASS_SYSADMIN FastEthernet 0/4 | 2023:2130:2787:7:60C7:44A3:3701:3701 | FE80::8:1 |
| CLASS_7 | FastEthernet 0 | SWITCH_CLASS_SYSADMIN FastEthernet 0/3 | 2023:2130:2787:7:CD55:939F:776B:776B | FE80::8:1 |
| WIRELESS IPV6 ADDRESS |  |  |  |  |
| WIRELESS PC | Wireless | SWITCH_INTERNET FastEthernet 0/1 | 2023:2130:2787:2:D40B:B9E6:AB44:9DB2 | FE80::2:1 |
| WIRELESS LAPTOP | Wireless | SWITCH_INTERNET FastEthernet 0/2 | 2023:2130:2787:2:89AE:4245:2621:2621 | FE80::2:1 |

| DEVICE | CONNECTED FROM | CONNECTED TO | IP ADDRESS | LINK-LOCAL |
| --- | --- | --- | --- | --- |
| MAIN_ROUTER | GigabitEthernet 0/0/0 | Switch_Server GigabitEthernet 0/2 | 2023:2130:2787:1::1 | FE80::1:1 |
|  | GigabitEthernet 0/0/1 | Switch_Internet GigabitEthernet 0/1 | 2023:2130:2787:2::1 | FE80::2:1 |
|  | Serial 0/1/0 | Cloud-PT Cloud Serial 0 | 2023:2130:2787:10::1 | FE80::5:1 |
|  | Serial 0/1/1 | Internal_Router Serial 0/2/0 | 2023:2130:2787:11::1 | FE80::5:2 |
|  | Serial 0/2/1 | Lab_Router Serial 0/1/0 | 2023:2130:2787:12::1 | FE80::5:3 |
| LAB_ROUTER | GigabitEthernet 0/0/0 | Switch_LAB1 GigabitEthernet 0/2 | 2023:2130:2787:3::1 | FE80::3:1 |
|  | GigabitEthernet 0/0/1 | Switch_LAB2 GigabitEthernet 0/2 | 2023:2130:2787:5::1 | FE80::4:1 |
|  | Serial0/1/0 | Main_Router Serial 0/2/0 | 2023:2130:2787:11::2 | FE80::5:4 |
| INTERNAL_ROUTER | GigabitEthernet 0/0/0 | Switch_Class_Sysadmin GigabitEthernet 0/1 | 2023:2130:2787:7::1 | FE80::8:1 |
|  | GigabitEthernet 0/0/1 | Tutor_Admin_Switch GigabitEthernet 0/1 | 2023:2130:2787:8::1 | FE80::9:1 |
|  | Serial0/2/0 | Main_Router Serial 0/1/1 | 2023:2130:2787:12::2 | FE80::5:6 |
| TM_ROUTER | GigabitEthernet 0/0/0 | Switch_ISP GigabitEthernet 0/1 | 2023:2130:2787:9::1 | FE80::10:1 |
|  | Serial 0/1/0 | Cloud-PT Cloud Serial 1 | 2023:2130:2787:13::1 | FE80::5:7 |
| FTP_SERVER | FastEthernet 0 | Switch_Server FastEthernet 0/1 | 2023:2130:2787:1::5 | 2023:2130:2787:1::1 |
| WEB_SERVER | FastEthernet 0 | Switch_Server FastEthernet 0/2 | 2023:2130:2787:1::4 | 2023:2130:2787:1::1 |
| DNS_SERVER | FastEthernet 0 | Switch_Server FastEthernet 0/3 | 2023:2130:2787:1::300 | 2023:2130:2787:1::1 |
| ISP_SERVER | FastEthernet 0 | Switch_Server GigabitEthernet 0/2 | 2023:2130:2787:9::5 | 2023:2130:2787:9::1 |

## ROUTER CONFIGURATION

Main Router

Router>enable

Router#configure terminal

Router(config)#hostname Main_Router

Main_Router(config)#ipv6 unicast-routing

Main_Router(config)#ipv6 dhcp pool STATEFUL_1

Main_Router(config-dhcpv6)#address prefix 2023:2130:2787:1::/64

Main_Router(config-dhcpv6)#dns-server 2023:2130:2787:1::300

Main_Router(config-dhcpv6)#domain-name Main_server.com

Main_Router(config-dhcpv6)#exit

Main_Router(config)#ipv6 dhcp pool STATEFUL_2

Main_Router(config-dhcpv6)#address prefix 2023:2130:2787:2::/64

Main_Router(config-dhcpv6)#dns-server 2023:2130:2787:1::300

Main_Router(config-dhcpv6)#domain-name Main_wifi.com

Main_Router(config-dhcpv6)#exit

Main_Router(config)#ipv6 router ospf 10

Main_Router(config-rtr)#router-id 1.1.1.1

Main_Router(config-rtr)#exit

Main_Router(config)#interface GigabitEthernet 0/0/0

Main_Router(config-if)#ipv6 address 2023:2130:2787:1::1/64

Main_Router(config-if)#ipv6 address FE80::1:1 link-local

Main_Router(config-if)#ipv6 nd managed-config-flag

Main_Router(config-if)#ipv6 dhcp server STATEFUL_1

Main_Router(config-if)#ipv6 ospf 10 area 0

Main_Router(config-if)#no shutdown

Main_Router(config-if)#exit

Main_Router(config)#interface GigabitEthernet 0/0/1

Main_Router(config-if)#ipv6 address 2023:2130:2787:2::1/64

Main_Router(config-if)#ipv6 address FE80::2:1 link-local

Main_Router(config-if)#ipv6 nd managed-config-flag

Main_Router(config-if)#ipv6 dhcp server STATEFUL_2

Main_Router(config-if)#ipv6 ospf 10 area 0

Main_Router(config-if)#no shutdown

Main_Router(config-if)#exit

Main_Router(config)#interface Serial 0/1/0

Main_Router(config-if)#ipv6 address 2023:2130:2787:10::1/64

Main_Router(config-if)#ipv6 address FE80::5:1 link-local

Main_Router(config-if)#ipv6 enable

Main_Router(config-if)#ipv6 ospf 10 area 0

Main_Router(config-if)#no shutdown

Main_Router(config-if)#exit

Main_Router(config)#interface serial 0/1/1

Main_Router(config-if)#ipv6 address 2023:2130:2787:11::1/64

Main_Router(config-if)#ipv6 address FE80::5:2 link-local

Main_Router(config-if)#ipv6 enable

Main_Router(config-if)#ipv6 ospf 10 area 0

Main_Router(config-if)#no shutdown

Main_Router(config-if)#exit

Main_Router(config)#interface Serial 0/2/1

Main_Router(config-if)#ipv6 address 2023:2130:2787:12::1/64

Main_Router(config-if)#ipv6 address FE80::5:3 link-local

Main_Router(config-if)#ipv6 enable

Main_Router(config-if)#ipv6 ospf 10 area 0

Main_Router(config-if)#no shutdown

Main_Router(config-if)#end

Main_Router#copy run start

Lab_Router

Router>enable

Router#configure terminal

Router(config)#hostname Lab_Router

Lab_Router(config)#ipv6 unicast-routing

Lab_Router(config)#ipv6 dhcp pool STATEFUL_3

Lab_Router(config-dhcpv6)#address prefix 2023:2130:2787:3::/64

Lab_Router(config-dhcpv6)#dns-server 2023:2130:2787:1::300

Lab_Router(config-dhcpv6)#domain-name Lab_1.com

Lab_Router(config-dhcpv6)#exit

Lab_Router(config)#ipv6 dhcp pool STATEFUL_4

Lab_Router(config-dhcpv6)#address prefix 2023:2130:2787:5::/64

Lab_Router(config-dhcpv6)#dns-server 2023:2130:2787:1::300

Lab_Router(config-dhcpv6)#domain-name Lab_2.com

Lab_Router(config-dhcpv6)#exit

Lab_Router(config)#ipv6 router ospf 10

Lab_Router(config-rtr)#router-id 2.2.2.2

Lab_Router(config-rtr)#exit

Lab_Router(config)#interface GigabitEthernet 0/0/0

Lab_Router(config-if)#ipv6 address 2023:2130:2787:3::1/64

Lab_Router(config-if)#ipv6 address FE80::3:1 link-local

Lab_Router(config-if)#ipv6 nd managed-config-flag

Lab_Router(config-if)#ipv6 dhcp server STATEFUL_3

Lab_Router(config-if)#ipv6 ospf 10 area 0

Lab_Router(config-if)#no shutdown

Lab_Router(config-if)#exit

Lab_Router(config-if)#interface GigabitEthernet 0/0/1

Lab_Router(config-if)#ipv6 address 2023:2130:2787:5::1/64

Lab_Router(config-if)#ipv6 address FE80::4:1 link-local

Lab_Router(config-if)#ipv6 nd managed-config-flag

Lab_Router(config-if)#ipv6 dhcp server STATEFUL_4

Lab_Router(config-if)#ipv6 ospf 10 area 0

Lab_Router(config-if)#no shutdown

Lab_Router(config-if)#exit

Lab_Router(config)#interface Serial 0/1/0

Lab_Router(config-if)#ipv6 address 2023:2130:2787:11::2/64

Lab_Router(config-if)#ipv6 address FE80::5:4 link-local

Lab_Router(config-if)#ipv6 enable

Lab_Router(config-if)#ipv6 ospf 10 area 0

Lab_Router(config-if)#no shutdown

Lab_Router(config-if)#end

Lab_Router#copy run start

Internal_Router

Router>enable

Router#configure terminal

Router(config)#hostname Internal_Router

Internal_Router(config)#ipv6 unicast-routing

Internal_Router(config)#ipv6 dhcp pool STATEFUL_5

Internal_Router(config-dhcpv6)#address prefix 2023:2130:2787:7::/64

Internal_Router(config-dhcpv6)#dns-server 2023:2130:2787:1::300

Internal_Router(config-dhcpv6)#domain-name Clasroom.com

Internal_Router(config-dhcpv6)#exit

Internal_Router(config)#ipv6 dhcp pool STATEFUL_6

Internal_Router(config-dhcpv6)#address prefix 2023:2130:2787:8::/64

Internal_Router(config-dhcpv6)#dns-server 2023:2130:2787:1::300

Internal_Router(config-dhcpv6)#domain-name Tutor_admin.com

Internal_Router(config-dhcpv6)#exit

Internal_Router(config)#ipv6 router ospf 10

Internal_Router(config-rtr)#router-id 3.3.3.3

Internal_Router(config-rtr)#exit

Internal_Router(config)#interface GigabitEthernet 0/0/0

Internal_Router(config-if)#ipv6 address 2023:2130:2787:7::1/64

Internal_Router(config-if)#ipv6 address FE80::8:1 link-local

Internal_Router(config-if)#ipv6 nd managed-config-flag

Internal_Router(config-if)#ipv6 dhcp server STATEFUL_5

Internal_Router(config-if)#ipv6 ospf 10 area 0

Internal_Router(config-if)#no shutdown

Internal_Router(config-if)#exit

Internal_Router(config)#interface GigabitEthernet 0/0/1

Internal_Router(config-if)#ipv6 address 2023:2130:2787:8::1/64

Internal_Router(config-if)#ipv6 address FE80::9:1 link-local

Internal_Router(config-if)#ipv6 nd managed-config-flag

Internal_Router(config-if)#ipv6 dhcp server STATEFUL_6

Internal_Router(config-if)#ipv6 ospf 10 area 0

Internal_Router(config-if)#no shutdown

Internal_Router(config-if)#exit

Internal_Router(config)#interface GigabitEthernet 0/0/0

Internal_Router(config-if)#ipv6 address 2023:2130:2787:7::1/64

Internal_Router(config-if)#ipv6 address FE80::8:1 link-local

Internal_Router(config-if)#ipv6 nd managed-config-flag

Internal_Router(config-if)#ipv6 dhcp server STATEFUL_5

Internal_Router(config-if)#ipv6 ospf 10 area 0

Internal_Router(config-if)#no shutdown

Internal_Router(config-if)#exit

Internal_Router(config)#interface Serial 0/2/0

Internal_Router(config-if)#ipv6 address 2023:2130:2787:12::2/64

Internal_Router(config-if)#ipv6 address FE80::5:6 link-local

Internal_Router(config-if)#ipv6 enable

Internal_Router(config-if)#ipv6 ospf 10 area 0

Internal_Router(config-if)#no shutdown

Internal_Router(config-if)#end

Internal_Router#copy run start

TM router

Router>enable

Router#configure terminal

Router(config)#hostname TM_Router

TM_Router(config)#ipv6 unicast-routing

TM_Router(config)#ipv6 dhcp pool STATEFUL_7

TM_Router(config-dhcpv6)#address prefix 2023:2130:2787:9::/64

TM_Router(config-dhcpv6)#dns-server 2023:2130:2787:1::300

TM_Router(config-dhcpv6)#domain-name TM_Server.com

TM_Router(config-dhcpv6)#exit

TM_Router(config)#ipv6 router ospf 10

TM_Router(config-rtr)#router-id 4.4.4.4

TM_Router(config-rtr)#exit

TM_Router(config)#interface GigabitEthernet 0/0/0

TM_Router(config-if)#ipv6 address 2023:2130:2787:9::1/64

TM_Router(config-if)#ipv6 address FE80::10:1 link-local

TM_Router(config-if)#ipv6 nd managed-config-flag

TM_Router(config-if)#ipv6 dhcp server STATEFUL_7

TM_Router(config-if)#ipv6 ospf 10 area 0

TM_Router(config-if)#no shutdown

TM_Router(config-if)#exit

TM_Router(config)#interface Serial 0/1/0

TM_Router(config-if)#ipv6 address 2023:2130:2787:13::1/64

TM_Router(config-if)#ipv6 address FE80::5:7 link-local

TM_Router(config-if)#ipv6 enable

TM_Router(config-if)#ipv6 ospf 10 area 0

TM_Router(config-if)#no shutdown

TM_Router(config-if)#end

TM_Router#copy run start

## IPV6 ROUTER CONFIGURATION

Main Router

Main_Router#show ipv6 route

IPv6 Routing Table - 19 entries

Codes: C - Connected, L - Local, S - Static, R - RIP, B - BGP

U - Per-user Static route, M - MIPv6

I1 - ISIS L1, I2 - ISIS L2, IA - ISIS interarea, IS - ISIS summary

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF intra, OI - OSPF inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2

D - EIGRP, EX - EIGRP external

O 2023:1110:319:1::/64 [110/65]

via FE80::5:6, Serial0/2/1

O 2023:1110:319:2::/64 [110/65]

via FE80::5:6, Serial0/2/1

C 2023:2130:2787:1::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L 2023:2130:2787:1::1/128 [0/0]

via GigabitEthernet0/0/0, receive

C 2023:2130:2787:2::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L 2023:2130:2787:2::1/128 [0/0]

via GigabitEthernet0/0/1, receive

O 2023:2130:2787:3::/64 [110/65]

via FE80::5:4, Serial0/1/1

O 2023:2130:2787:5::/64 [110/65]

via FE80::5:4, Serial0/1/1

O 2023:2130:2787:7::/64 [110/65]

via FE80::5:6, Serial0/2/1

O 2023:2130:2787:8::/64 [110/65]

via FE80::5:6, Serial0/2/1

O 2023:2130:2787:9::/64 [110/65]

via FE80::5:7, Serial0/1/0

C 2023:2130:2787:10::/64 [0/0]

via Serial0/1/0, directly connected

L 2023:2130:2787:10::1/128 [0/0]

via Serial0/1/0, receive

C 2023:2130:2787:11::/64 [0/0]

via Serial0/1/1, directly connected

L 2023:2130:2787:11::1/128 [0/0]

via Serial0/1/1, receive

C 2023:2130:2787:12::/64 [0/0]

via Serial0/2/1, directly connected

L 2023:2130:2787:12::1/128 [0/0]

via Serial0/2/1, receive

O 2023:2130:2787:13::/64 [110/64]

via

L FF00::/8 [0/0]

via Null0, receive

Lab Router

Lab_Router#show ipv6 route

IPv6 Routing Table - 17 entries

Codes: C - Connected, L - Local, S - Static, R - RIP, B - BGP

U - Per-user Static route, M - MIPv6

I1 - ISIS L1, I2 - ISIS L2, IA - ISIS interarea, IS - ISIS summary

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF intra, OI - OSPF inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2

D - EIGRP, EX - EIGRP external

O 2023:1110:319:1::/64 [110/129]

via FE80::5:2, Serial0/1/0

O 2023:1110:319:2::/64 [110/129]

via FE80::5:2, Serial0/1/0

O 2023:2130:2787:1::/64 [110/65]

via FE80::5:2, Serial0/1/0

O 2023:2130:2787:2::/64 [110/65]

via FE80::5:2, Serial0/1/0

C 2023:2130:2787:3::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L 2023:2130:2787:3::1/128 [0/0]

via GigabitEthernet0/0/0, receive

C 2023:2130:2787:5::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L 2023:2130:2787:5::1/128 [0/0]

via GigabitEthernet0/0/1, receive

O 2023:2130:2787:7::/64 [110/129]

via FE80::5:2, Serial0/1/0

O 2023:2130:2787:8::/64 [110/129]

via FE80::5:2, Serial0/1/0

O 2023:2130:2787:9::/64 [110/129]

via FE80::5:2, Serial0/1/0

O 2023:2130:2787:10::/64 [110/128]

via FE80::5:2, Serial0/1/0

C 2023:2130:2787:11::/64 [0/0]

via Serial0/1/0, directly connected

L 2023:2130:2787:11::2/128 [0/0]

via Serial0/1/0, receive

O 2023:2130:2787:12::/64 [110/128]

via FE80::5:2, Serial0/1/0

O 2023:2130:2787:13::/64 [110/128]

via FE80::5:2, Serial0/1/0

L FF00::/8 [0/0]

via Null0, receive

Internal Router

Internal_Router#show ipv6 route

IPv6 Routing Table - 19 entries

Codes: C - Connected, L - Local, S - Static, R - RIP, B - BGP

U - Per-user Static route, M - MIPv6

I1 - ISIS L1, I2 - ISIS L2, IA - ISIS interarea, IS - ISIS summary

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF intra, OI - OSPF inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2

D - EIGRP, EX - EIGRP external

C 2023:1110:319:1::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L 2023:1110:319:1::1/128 [0/0]

via GigabitEthernet0/0/0, receive

C 2023:1110:319:2::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L 2023:1110:319:2::1/128 [0/0]

via GigabitEthernet0/0/1, receive

O 2023:2130:2787:1::/64 [110/65]

via FE80::5:3, Serial0/2/0

O 2023:2130:2787:2::/64 [110/65]

via FE80::5:3, Serial0/2/0

O 2023:2130:2787:3::/64 [110/129]

via FE80::5:3, Serial0/2/0

O 2023:2130:2787:5::/64 [110/129]

via FE80::5:3, Serial0/2/0

C 2023:2130:2787:7::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L 2023:2130:2787:7::1/128 [0/0]

via GigabitEthernet0/0/0, receive

C 2023:2130:2787:8::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L 2023:2130:2787:8::1/128 [0/0]

via GigabitEthernet0/0/1, receive

O 2023:2130:2787:9::/64 [110/129]

via FE80::5:3, Serial0/2/0

O 2023:2130:2787:10::/64 [110/128]

via FE80::5:3, Serial0/2/0

O 2023:2130:2787:11::/64 [110/128]

via FE80::5:3, Serial0/2/0

C 2023:2130:2787:12::/64 [0/0]

via Serial0/2/0, directly connected

L 2023:2130:2787:12::2/128 [0/0]

via Serial0/2/0, receive

O 2023:2130:2787:13::/64 [110/128]

via FE80::5:3, Serial0/2/0

L FF00::/8 [0/0]

via Null0, receive

TM Router

TM_Router#show ipv6 route

IPv6 Routing Table - 16 entries

Codes: C - Connected, L - Local, S - Static, R - RIP, B - BGP

U - Per-user Static route, M - MIPv6

I1 - ISIS L1, I2 - ISIS L2, IA - ISIS interarea, IS - ISIS summary

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF intra, OI - OSPF inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2

D - EIGRP, EX - EIGRP external

O 2023:1110:319:1::/64 [110/129]

via FE80::5:1, Serial0/1/0

O 2023:1110:319:2::/64 [110/129]

via FE80::5:1, Serial0/1/0

O 2023:2130:2787:1::/64 [110/65]

via FE80::5:1, Serial0/1/0

O 2023:2130:2787:2::/64 [110/65]

via FE80::5:1, Serial0/1/0

O 2023:2130:2787:3::/64 [110/129]

via FE80::5:1, Serial0/1/0

O 2023:2130:2787:5::/64 [110/129]

via FE80::5:1, Serial0/1/0

O 2023:2130:2787:7::/64 [110/129]

via FE80::5:1, Serial0/1/0

O 2023:2130:2787:8::/64 [110/129]

via FE80::5:1, Serial0/1/0

C 2023:2130:2787:9::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L 2023:2130:2787:9::1/128 [0/0]

via GigabitEthernet0/0/0, receive

O 2023:2130:2787:10::/64 [110/64]

via

O 2023:2130:2787:11::/64 [110/128]

via FE80::5:1, Serial0/1/0

O 2023:2130:2787:12::/64 [110/128]

via FE80::5:1, Serial0/1/0

C 2023:2130:2787:13::/64 [0/0]

via Serial0/1/0, directly connected

L 2023:2130:2787:13::1/128 [0/0]

via Serial0/1/0, receive

L FF00::/8 [0/0]

via Null0, receive

## DHCP SERVER CONFIGURATION

Main_Router

Main_Router>enable

Main_Router#configure terminal

Main_Router(config)#ipv6 unicast-routing

Main_Router(config)#ipv6 dhcp pool STATEFUL_1

Main_Router(config-dhcpv6)#address prefix 2023:2130:2787:1::/64

Main_Router(config-dhcpv6)#dns-server 2023:2130:2787:1::300

Main_Router(config-dhcpv6)#domain-name Main_server.com

Main_Router(config-dhcpv6)#exit

Main_Router(config)#ipv6 dhcp pool STATEFUL_2

Main_Router(config-dhcpv6)#address prefix 2023:2130:2787:2::/64

Main_Router(config-dhcpv6)#dns-server 2023:2130:2787:1::300

Main_Router(config-dhcpv6)#domain-name Main_wifi.com

Main_Router(config-dhcpv6)#exit

Main_Router(config)#interface GigabitEthernet 0/0/0

Main_Router(config-if)#ipv6 nd managed-config-flag

Main_Router(config-if)#ipv6 dhcp server STATEFUL_1

Main_Router(config-if)#no shutdown

Main_Router(config-if)#exit

Main_Router(config)#interface GigabitEthernet 0/0/1

Main_Router(config-if)#ipv6 nd managed-config-flag

Main_Router(config-if)#ipv6 dhcp server STATEFUL_2

Main_Router(config-if)#no shutdown

Main_Router(config-if)#exit

Lab_Router

Lab_Router>enable

Lab_Router#configure terminal

Lab_Router(config)#ipv6 unicast-routing

Lab_Router(config)#ipv6 dhcp pool STATEFUL_3

Lab_Router(config-dhcpv6)#address prefix 2023:2130:2787:3::/64

Lab_Router(config-dhcpv6)#dns-server 2023:2130:2787:1::300

Lab_Router(config-dhcpv6)#domain-name Lab_1.com

Lab_Router(config-dhcpv6)#exit

Lab_Router(config)#ipv6 dhcp pool STATEFUL_4

Lab_Router(config-dhcpv6)#address prefix 2023:2130:2787:5::/64

Lab_Router(config-dhcpv6)#dns-server 2023:2130:2787:1::300

Lab_Router(config-dhcpv6)#domain-name Lab_2.com

Lab_Router(config-dhcpv6)#exit

Lab_Router(config)#interface GigabitEthernet 0/0/0

Lab_Router(config-if)#ipv6 nd managed-config-flag

Lab_Router(config-if)#ipv6 dhcp server STATEFUL_3

Lab_Router(config-if)#no shutdown

Lab_Router(config-if)#exit

Lab_Router(config-if)#interface GigabitEthernet 0/0/1

Lab_Router(config-if)#ipv6 nd managed-config-flag

Lab_Router(config-if)#ipv6 dhcp server STATEFUL_4

Lab_Router(config-if)#no shutdown

Lab_Router(config-if)#exit

Internal_Router

Internal_Router>enable

Internal_Router#configure terminal

Internal_Router(config)#ipv6 unicast-routing

Internal_Router(config)#ipv6 dhcp pool STATEFUL_5

Internal_Router(config-dhcpv6)#address prefix 2023:2130:2787:7::/64

Internal_Router(config-dhcpv6)#dns-server 2023:2130:2787:1::300

Internal_Router(config-dhcpv6)#domain-name Clasroom.com

Internal_Router(config-dhcpv6)#exit

Internal_Router(config)#ipv6 dhcp pool STATEFUL_6

Internal_Router(config-dhcpv6)#address prefix 2023:2130:2787:8::/64

Internal_Router(config-dhcpv6)#dns-server 2023:2130:2787:1::300

Internal_Router(config-dhcpv6)#domain-name Tutor_admin.com

Internal_Router(config-dhcpv6)#exit

Internal_Router(config)#interface GigabitEthernet 0/0/0

Internal_Router(config-if)#ipv6 nd managed-config-flag

Internal_Router(config-if)#ipv6 dhcp server STATEFUL_5

Internal_Router(config-if)#no shutdown

Internal_Router(config-if)#exit

Internal_Router(config)#interface GigabitEthernet 0/0/1

Internal_Router(config-if)#ipv6 nd managed-config-flag

Internal_Router(config-if)#ipv6 dhcp server STATEFUL_6

Internal_Router(config-if)#no shutdown

Internal_Router(config-if)#exit

TM Router

TM_Router>enable

TM_Router#configure terminal

TM_Router(config)#ipv6 unicast-routing

TM_Router(config)#ipv6 dhcp pool STATEFUL_7

TM_Router(config-dhcpv6)#address prefix 2023:2130:2787:9::/64

TM_Router(config-dhcpv6)#dns-server 2023:2130:2787:1::300

TM_Router(config-dhcpv6)#domain-name TM_Server.com

TM_Router(config-dhcpv6)#exit

TM_Router(config)#interface GigabitEthernet 0/0/0

TM_Router(config-if)#ipv6 nd managed-config-flag

TM_Router(config-if)#ipv6 dhcp server STATEFUL_7

TM_Router(config-if)#no shutdown

TM_Router(config-if)#exit

## OSPF CONFIGURATION

Main Router

Main_Router>enable

Main_Router#configure terminal

Main_Router(config)#ipv6 router ospf 10

Main_Router(config-rtr)#router-id 1.1.1.1

Main_Router(config-rtr)#exit

Main_Router(config)#interface GigabitEthernet 0/0/0

Main_Router(config-if)#ipv6 ospf 10 area 0

Main_Router(config-if)#no shutdown

Main_Router(config-if)#exit

Main_Router(config)#interface GigabitEthernet 0/0/1

Main_Router(config-if)#ipv6 ospf 10 area 0

Main_Router(config-if)#no shutdown

Main_Router(config-if)#exit

Main_Router(config)#interface Serial 0/1/0

Main_Router(config-if)#ipv6 ospf 10 area 0

Main_Router(config-if)#no shutdown

Main_Router(config-if)#exit

Main_Router(config)#interface serial 0/1/1

Main_Router(config-if)#ipv6 ospf 10 area 0

Main_Router(config-if)#no shutdown

Main_Router(config-if)#exit

Main_Router(config)#interface Serial 0/2/1

Main_Router(config-if)#ipv6 ospf 10 area 0

Main_Router(config-if)#no shutdown

Main_Router(config-if)#exit

Lab Router

Lab_Router>enable

Lab_Router#configure terminal

Lab_Router(config)#ipv6 router ospf 10

Lab_Router(config-rtr)#router-id 2.2.2.2

Lab_Router(config-rtr)#exit

Lab_Router(config)#interface GigabitEthernet 0/0/0

Lab_Router(config-if)#ipv6 ospf 10 area 0

Lab_Router(config-if)#no shutdown

Lab_Router(config-if)#exit

Lab_Router(config-if)#interface GigabitEthernet 0/0/1

Lab_Router(config-if)#ipv6 ospf 10 area 0

Lab_Router(config-if)#no shutdown

Lab_Router(config-if)#exit

Lab_Router(config)#interface Serial 0/1/0

Lab_Router(config-if)#ipv6 ospf 10 area 0

Lab_Router(config-if)#no shutdown

Lab_Router(config-if)#end

Internal Router

Internal_Router>enable

Internal_Router#configure terminal

Internal_Router(config)#ipv6 router ospf 10

Internal_Router(config-rtr)#router-id 3.3.3.3

Internal_Router(config-rtr)#exit

Internal_Router(config)#interface GigabitEthernet 0/0/0

Internal_Router(config-if)#ipv6 ospf 10 area 0

Internal_Router(config-if)#no shutdown

Internal_Router(config-if)#exit

Internal_Router(config)#interface GigabitEthernet 0/0/1

Internal_Router(config-if)#ipv6 ospf 10 area 0

Internal_Router(config-if)#no shutdown

Internal_Router(config-if)#exit

Internal_Router(config)#interface GigabitEthernet 0/0/0

Internal_Router(config-if)#ipv6 ospf 10 area 0

Internal_Router(config-if)#no shutdown

Internal_Router(config-if)#exit

Internal_Router(config)#interface Serial 0/2/0

Internal_Router(config-if)#ipv6 ospf 10 area 0

Internal_Router(config-if)#no shutdown

Internal_Router(config-if)#end

TM Router

TM_Router>enable

TM_Router#configure terminal

TM_Router(config)#ipv6 router ospf 10

TM_Router(config-rtr)#router-id 4.4.4.4

TM_Router(config-rtr)#exit

TM_Router(config)#interface GigabitEthernet 0/0/0

TM_Router(config-if)#ipv6 ospf 10 area 0

TM_Router(config-if)#no shutdown

TM_Router(config-if)#exit

TM_Router(config)#interface Serial 0/1/0

TM_Router(config-if)#ipv6 ospf 10 area 0

TM_Router(config-if)#no shutdown

TM_Router(config-if)#end

## CLOUD AND ISP ROUTER CONFIGURATION

Main Router

Main_Router>enable

Main_Router#configure terminal

Main_Router(config)#int se0/1/0

Main_Router(config-if)#encap frame-relay

Main_Router(config-if)#frame-relay map ipv6 2023:2130:2787:13::1 100

Main_Router(config-if)#frame-relay map ipv6 FE80::5:7 100 broadcast

Main_Router(config-if)#ipv6 add FE80::5:1 link-local

Main_Router(config-if)#ipv6 ospf neighbor FE80::5:7

Main_Router(config-if)#no shut

Main_Router(config-if)#end

Main_Router#copy run start

TM Router

TM_Router>enable

TM_Router#configure terminal

TM_Router(config)#int se0/1/0

TM_Router(config-if)#encap frame-relay

TM_Router(config-if)#frame-relay map ipv6 2023:2130:2787:10::1 200

TM_Router(config-if)#frame-relay map ipv6 FE80::5:1 200 broadcast

TM_Router(config-if)#ipv6 add FE80::5:7 link-local

TM_Router(config-if)#ipv6 ospf neighbor FE80::5:1

TM_Router(config-if)#no shut

TM_Router(config-if)#end

TM_Router#copy run start

## ACCESS CONTROL LIST

### DENY INTERNET ACCESS TO ADMIN AND TUTOR ROOM

DENY INTERNET ACCESS FOR TUTOR AND ADMIN ROOM

Internal_Router>enable

Internal_Router#configure terminal

Internal_Router(config)#IPv6 access-list Tutor_admin

Internal_Router(config-ipv6-acl)#deny tcp 2023:2130:2787:8::1/64 2023:2130:2787:9::1/64 eq www

Internal_Router(config-ipv6-acl)#deny tcp 2023:2130:2787:8::1/64 2023:2130:2787:9::1/64 eq 443

Internal_Router(config-ipv6-acl)#permit icmp any any

Internal_Router(config-ipv6-acl)#permit ipv6 any any

Internal_Router(config-ipv6-acl)#permit udp any any

Internal_Router(config-ipv6-acl)#exit

Internal_Router(config)#interface Gigabitethernet 0/0/1

Internal_Router(config-if)#ipv6 traffic-filter Tutor_admin in

Internal_Router(config-if)#end

Internal_Router#copy run start

### DENY INTERNET ACCESS TO WIRELESS NETWORK

DENY INTERNET ACCESS FOR WIRELESS DEVICE

Main_Router>enable

Main_Router#configure terminal

Main_Router(config)#IPv6 access-list Wireless

Main_Router(config-ipv6-acl)#deny tcp 2023:2130:2787:2::1/64 2023:2130:2787:9::1/64 eq www

Main_Router(config-ipv6-acl)#deny tcp 2023:2130:2787:2::1/64 2023:2130:2787:9::1/64 eq 443

Main_Router(config-ipv6-acl)#permit icmp any any

Main_Router(config-ipv6-acl)#permit ipv6 any any

Main_Router(config-ipv6-acl)#permit udp any any

Main_Router(config-ipv6-acl)#exit

Main_Router(config)#interface Gigabitethernet 0/0/1

Main_Router(config-if)#ipv6 traffic-filter Wireless in

Main_Router(config-if)#end

Main_Router#copy run start

## WEB SERVER CONFIGURATION

Figure 10.1. IP Configuratin for web server

Figure 10.2. HTTP configuration for web server

Figure 10.2 Accessing web server from PC6 tutor room

## FTP SERVER CONFIGURATION

Figure 11.1: IP Configuration of FTP Server

Figure 11.2. FTP Service Configuration

Figure 11.3 : Creating a test file using PC8_Lab1

Figure 11.4 : Uploading test file named Test.txt to the File Server from PC8_LAB1

Figure 11.5 : Downloading Test.txt file to PC18_lab2

Figure 11.6 : Test.txt file is downloaded to PC18_LAB2

Figure 11.7 : Contents of Test.txt file can be opened and read using PC18_LAB2

## DNS SERVER CONFIGURATION

Figure 12.1.  IP configuration for DNS server

Figure 12.2. DNS services configuraion for DNS Server

Figure 12.3 Accessing DNS server from PC3 classroom

## TM SERVER CONFIGURATION

Figure 13.1 : IP Configuration of TM Server(ISP Server)

Figure 13.2 : PC25_LAB2_INSTRUCTOR can access www.google.com

Figure 13.3 : PC_TUTOR2 not allowed access to www.google.com

Figure 13.4 : PC_CLASS_6 allowed access to www.google.com

Figure 13.5 : ADMIN_4 not allowed access to www.google.com

Figure 13.6 : Shows the link between Serial0 and Serial1 in Frame Relay

Figure 13.7 : Shows interface for Serial1

Figure 13.8: Shows interface for Serial1

## CONNECTIVITY STATUS

Figure 14.1.  Ping from wireless laptop to LAb2_PC22

Figure 14.2  ping from wireless laptop to LAB1_PC22

Figure 14.3 Ping from wireless laptop to ADMIN_2

Figure 14.4.  Ping from wireless laptop to Sysadmin (management)

Figure 14.5. Ping wireless laptop to classroom PC4